1SECTION 10 2116

SOLID PLASTIC TOILET COMPARTMENTS

This section includes editing notes to assist the user in editing the section to suit project requirements. These notes are included as hidden text, and can be revealed or hidden by one of the following methods:

Microsoft Word 2010: Display the FILE tab on the ribbon, click OPTIONS, then on left menu click on DISPLAY. Under ALWAYS SHOW THESE select or deselect HIDDEN TEXT.

Microsoft Word 2007: Click the OFFICE button, select WORD OPTIONS, select DISPLAY, then select or deselect the HIDDEN TEXT option.

Corel WordPerfect: From the pull-down menus select VIEW, then select or deselect the HIDDEN TEXT option.

This master specification section has been prepared by AJW Architectural Products for use in the preparation of a project specification section covering floor mounted, overhead braced, floor to ceiling, or ceiling hung solid plastic toilet compartments and wall or floor mounted urinal screens.

This specification is a part of the SpexPlus™ system, which comprises a full architectural master specification that can be used to specify all project requirements.

Other guide specifications are available from AJW covering the following products:

Section 10 1100 - Visual Display Units

Section 10 2114 - Stainless Steel Toilet Compartments

Section 10 2813 - Toilet Accessories

Section 10 5113 - Metal Lockers

The following should be noted in using this specification:

Hypertext links to specific websites are included after manufacturer names and names of organizations whose standards are referenced within the text, to assist in product selection and further research. Hypertext links are contained in parenthesis and shown in blue, e.g.:

(www.spexplus.net)

Optional text requiring a selection by the user is enclosed within brackets, e.g.: "Section [09 0000.] [_____.]"

Items requiring user input are enclosed within brackets, e.g.: "Section [_____ - ________]."

Optional paragraphs are separated by an "OR" statement, e.g.:

**** OR ****

Sustainable requirements are included for projects requiring LEED certification, and are included as green text. For additional information on LEED, visit the U.S. Green Building Council website at www.usgbc.org.

For assistance on the use of the products in this section, contact AJW Architectural Products by calling 845-562-3332, email at specs@ajw.com, or visit their website at www.ajw.com.

For assistance with obtaining or using the SpexPlus™ Master Specification System contact SpexPlus by calling 1-888-877-SPEX (1-888-877-7739), by email at chaney@spexplus.net, or visit our website at www.spexplus.net.

- GENERAL

  - SUMMARY

Edit the following paragraphs to include only those items specified in this section.

    - Section Includes:

      - Solid plastic toilet partitions.

      - Solid plastic urinal screens.

      - Solid plastic privacy/entry screens.

      - Solid plastic compartment [fronts.] [enclosures.]

Coordinate the following paragraphs with other sections in the project manual.

    - Related Sections:

      - Division 01: Administrative, procedural, and temporary work requirements.

      - Section 10 2813 - Toilet Accessories.

  - REFERENCES

In the following paragraphs, retain only those reference standards that are used elsewhere in this section.

    - ASTM International (ASTM) (www.astm.org):

      - A167 - Standard Specification for Stainless and Heat Resisting Chromium Nickel Steel Plate, Sheet and Strip.

      - B221 - Standard Specification for Aluminum and Aluminum-Alloy Extruded Bars, Rods, Wire, Profiles, and Tubes.

      - E84 - Standard Test Method for Surface Burning Characteristics of Building Materials.

    - International Code Council (ICC) (www.iccsafe.com) - International Building Code.

  - SYSTEM DESCRIPTION

    - Compartment Mounting Styles:

      - Toilet partitions: [Floor mounted, overhead braced.] [Floor to ceiling.] [Ceiling hung.]

      - Urinal screens: [Wall mounted.] [Wall mounted, floor supported.]

    - Doors and Panels: 55 inches high, mounted 14 inches above finished floor.

  - SUBMITTALS

Limiting submittals to only those actually required helps to minimize liability arising from the review of submittals. Minimize submittals on smaller, less complex projects.

Include the following for submission of shop drawings, product data, and samples for the Architect's review.

    - Submittals for Review:

      - Shop Drawings: Include layout, dimensions, materials, panel construction, finishes, hardware, and accessories.

      - Samples: [3 x 3] [__ x __] inch panel samples [showing available colors.] [in each color.]

Include the following for submission of sustainable design submittals.

    - Sustainable Design Submittals:

      - Recycled Content: Certify percentages of post-consumer and pre-consumer recycled content.

      - Regional Materials: Certify distance between manufacturer and project and between manufacturer and extraction or harvest point in miles.

  - QUALITY ASSURANCE

The following paragraph specifies a minimum level of experience required of the parties performing the work of this section. Retain if required, and edit to suit project requirements.

    - Installer Qualifications: Minimum [2] [__] years [documented] experience in work of this Section.

    - Fire Resistance Ratings:

      - Maximum flame spread/smoke developed rating of 75/450, tested to ASTM E84.

      - IBC Class B fire hazard classification.

  - DELIVERY, STORAGE AND HANDLING

    - Deliver panels with temporary removable plastic protective coverings; do not remove until ready for final cleaning.

  - WARRANTIES

    - Provide manufacturer’s 25 year warranty providing coverage against breakage, corrosion, and delamination of solid plastic panels.

- PRODUCTS

  - MANUFACTURERS

    - Contract Documents are based on products by AJW Architectural Products, 509 Temple Hill Road, New Windsor, NY 12553, 845-562-3332, www.ajw.com, specs@ajw.com.

Edit the following to indicate whether or not substitutions will be permitted for the products in this section.

    - Substitutions: [Under provisions of Division 01.] [Not permitted.]

  - MATERIALS

    - Plastic Doors, Panels, Pilasters, and Urinal Screens:

      - HDPE (High Density Polyethylene), integrally colored, fabricated from extruded polymer resin forming single, one piece, solid construction sheet.

      - Waterproof, non-absorbent, with self-lubricating surface resistant to marks from pens, markers, and paints.

      - Free from urea-formaldehyde resins.

      - No volatile organic compounds (VOC) emissions.

      - Recycled content: Minimum 25 percent, classified as pre-consumer.

    - Aluminum Extrusions: ASTM B221, 6463-T5 alloy.

    - Stainless Steel: ASTM A167.

  - HARDWARE

Include the following for aluminum wrap-around type hinges.

    - Hinges:

      - Aluminum wrap-around type, 8 inches long, fabricated from heavy duty extruded aluminum, with wrap around flanges through bolted through doors and pilasters with stainless steel tamper resistant Torx head fasteners.

      - Hinges operate with field adjustable nylon cams, adjustable in 30 degree increments.

**** OR ****

Include the following for integral type hinges.

    - Hinges:

      - Integral type, pilaster machined to accept door and hinge mechanism.

      - Hinge mechanism: Two piece 1/2 inch diameter nylon pin with “cam action” and 3/16 inch stainless steel pin inserted into lower section on pilaster and door.

      - One piece 1/2 inch diameter, 4 inch long nylon pin inserted in top section of pilaster and door.

      - Field adjustable to varying degrees of open or set to closed position at rest.

**** OR ****

Include the following for continuous type hinges.

    - Hinges: Continuous [stainless steel helix type,] [aluminum spring loaded type,] 54 inches long, secured full length of door, fastened with stainless steel tamper resistant Torx head fasteners.

    - Door Strike and Keeper:

Edit the following to indicate desired type of strike and keeper.

      - [6 inches long, fabricated from heavy duty extruded aluminum] [3-1/2 inches long, cast stainless steel] with wrap around flanges.

      - Integral extruded black vinyl bumper.

Edit the following to indicate desired type of slide latch and housing.

    - Slide Latch and Housing: [Fabricated from heavy duty aluminum, mitered for emergency access.] [Cast stainless steel, configured to allow emergency access.]

    - Coat Hook/Bumpers: Chrome plated Zamac.

    - Door Pulls: Chrome plated Zamac.

Edit the following to indicate desired type of pilaster shoe.

    - Pilaster Shoes: [Stainless steel, 20 gauge,] [Plastic, one piece molded HDPE,] 3 inches high.

Include the following for aluminum stirrup type brackets.

    - Wall Brackets: Aluminum stirrup type, 1-1/2 inches, fabricated from heavy duty extruded aluminum fastened to pilasters and panels with stainless steel tamper resistant Torx head fasteners, three brackets per panel connection.

**** OR ****

Include the following for stainless steel stirrup type brackets.

    - Wall Brackets: Stainless steel stirrup type 1-1/2 inch, 14 gauge stamped stainless steel fastened to pilasters and panels with stainless steel tamper resistant Torx head fasteners, three brackets per panel connection.

**** OR ****

Include the following for continuous type brackets. Edit to indicate desired bracket material.

    - Wall Brackets: Continuous type, [heavy duty extruded aluminum,] [14 gauge stamped stainless steel,] [extruded PVC,] 54-1/2 inch long, secured full length of panel with stainless steel tamper resistant Torx head fasteners.

    - Head Rail: Fabricated from heavy duty extruded aluminum, anti grip design, fastened to pilaster and secured to adjacent construction with stainless steel brackets.

  - FABRICATION

In the following paragraph a heat sink is optional.

    - Doors, Panels, and Pilasters: 1 inch thick with edges machined to rounded radius [and aluminum heat sink fastened to bottom edges].

    - Partition Dividing Panels and Doors: 55 inches high, mounted 14 inches above finished floor.

    - Pilasters: 82 inches high, fastened to finished floor with 3 inch high pilaster shoe using stainless steel tamper resistant Torx head fasteners.

  - FINISHES

Edit the following to suit project requirements. Refer to current AJW Architectural Product information for available colors.

    - Solid Plastic Panels: [____] color [to be selected from manufacturer's full color range].

    - Aluminum:

      - Slide bolt and handle: Black anodized.

      - Other components: Clear anodized.

    - Stainless Steel: No. 4 satin.

- EXECUTION

  - INSTALLATION

    - Install in accordance with manufacturer’s instructions and approved Shop Drawings.

    - Set compartments straight, plumb, level, and aligned.

    - Provide uniform clearances at vertical edges of doors from top to bottom

    - Attach panel and head rail brackets to walls using appropriate anchor devices.

    - Adjust for floor variations with screw jack integral in pilasters. Conceal floor fastenings with pilaster shoes.

Edit the following to suit project requirements.

    - Equip doors with two hinges, door strike/keeper, slide latch, door pull, and coat hook/bumper. [Provide one additional bumper/hook on inside of outswinging doors.]

    - Not Acceptable: Evidence of cutting, drilling, or patching on exposed surfaces.

  - ADJUSTING

    - Adjust hardware for proper operation.

Edit the following to suit project requirements.

    - Adjust door hinges to hold door [closed] [open [30] [__] degrees] when not latched.

END OF SECTION